# Project Overview: Marvin Sim Chair

Haptic Sim-Chair-Based Teleoperation System for Autonomous Vehicle Platforms

Cal Poly Pomona Autonomous Vehicles Laboratory (AVL)

Team: Ryan Simpson (lead), Tyler Carrasco, Yi-Szen Tang, Kevin Xu, Justin Dang

Mentor: Dr. Behnam Bahr

# The problem

Remote drivers lose the cues that make driving intuitive:

Peripheral vision and depth from being in the seat

Body motion and the feel of acceleration, braking, and cornering

Confident, precise control without those sensory inputs

Flat single-camera teleoperation is not enough for hazardous environments or autonomy safety intervention.

# The goal

Give the operator a human level of sight and realistic physical feedback while driving Marvin (go-kart) from a D-BOX sim chair over any internet link:

Vision: three ZED X cameras → three panoramic monitors

Control: Thrustmaster wheel/pedals → differential drive in real time

Haptics: vehicle IMU accelerations → D-BOX chair motion (path ready; end-to-end demo verification still required)

All links run over Tailscale VPN (no line-of-sight radio required).

# Platform at a glance

| Side | Hardware |
| --- | --- |
| Operator (Windows sim-chair) | Thrustmaster wheel/pedals, D-BOX motion chair, 3 monitors (Surround: x=0 / 1920 / 3840) |
| Vehicle (Jetson Orin dinosaur) | 3× ZED X GMSL, Xsens IMU, Teensy 4.1 → REV Spark MAX, Velodyne LiDAR, NTRIP GPS |

Extension of the AVL IGVC robotics platform: teleop code lives outside /home/dinosaur/IGVC/ (never modify that tree).

# Current status (lab-accurate)

| Capability | Status |
| --- | --- |
| Sim-chair wheel/pedal → kart motors | Working |
| Three live JPEG camera streams on monitors | Working (UDP primary ~1280×720 @ 30) |
| D-BOX SDK integrated on Windows | Working |
| Vehicle IMU → D-BOX chair cueing | Script ready (imu_bridge.py); E2E chair motion needs demo verification |
| Motor drive via sim chair | Working with --duty (UL/UR); velocity mode often purple |
| H.265 primary video path | Experimental; UDP JPEG is current primary |

# Near-term objectives

Median glass-to-glass camera latency under 150 ms (on direct Tailscale)

Smooth remote driving feel comparable to sitting in the vehicle

Cameras + controls + haptic chair running together in one demo

# Technologies

Jetson Orin · ROS 2 Humble · CycloneDDS · ZED X · Xsens MTi · D-BOX LiveMotion · Tailscale · SDL2 · Python / C++ · Teensy 4.1 · REV Spark MAX

See also: PROJECT_OVERVIEW.md · ABSTRACT.md · CARS poster pack