# Demo Summary: Marvin Sim Chair

Updated: 2026-08-07

What a live lab demo shows today, how to start it, and what not to over-claim.

# What the audience sees

Operator station: D-BOX chair, Thrustmaster wheel/pedals, three panoramic monitors

Panoramic video: left / front / right ZED views (UDP JPEG)

Remote drive: wheel/pedals → UDP 5006 → duty-mode vehicle controller → motors

Status: optional control-status window (UDP 5008)

Vehicle: Marvin with three ZED X cameras and Jetson Orin

# Current demo path

| Channel | Path | Status |
| --- | --- | --- |
| Control | Wheel → UDP 5006 → orin_vehicle_controller.py --duty → Teensy | Working |
| Video | ZED → orin_camera_sender_udp.py → UDP JPEG 9001-9003 → windows_camera_receiver_udp.py | Working |
| Haptics | Local input→D-BOX via sender exe | Available |
| Haptics | Vehicle IMU → UDP 5007 → washout → D-BOX | Do not claim unless verified that day |

Do not claim unless checked that day: H.265 primary, glass-to-glass <150 ms, full IMU→chair washout, perfect sustained motors.

# Minimal launch (demo day)

## Jetson

bash ~/Documents/SimChair_Teleop/orin_launch_all.sh

Preflight clears ZED contention; starts UDP cameras, --duty drive, IMU bridge.

Stop: tmux kill-session -t teleop then sudo systemctl start avros-webui

## Windows

cd "C:\Users\VRX 226\cursor-test\windows"
.\start_all.bat
# default VIDEO_MODE=udp

Drive-only borrow: bash ~/Documents/SimChair_Teleop/simchair_drive.sh (Ctrl+C restores WebUI).

# Demo talk track (≈60 s)

Problem: remote drivers lose FOV and motion cues

Solution: three-camera panorama + wheel/pedals over Tailscale + D-BOX

Show monitors (UDP JPEG path)

Show kart respond to wheel

Close: next = vehicle-IMU motion cueing + H.265 / lower latency

See also: LAUNCH_PROCEDURE.md · LAUNCH.txt