# Technical Architecture: Marvin Sim Chair

Updated: 2026-08-07

Three bidirectional channels over Tailscale connect the sim-chair Windows PC to Marvin’s Jetson Orin.

flowchart LR
  subgraph operator [Operator Windows]
    Wheel[Thrustmaster SDL2]
    Sender[simchair_sender_dbox.exe]
    Cams[windows_camera_receiver_udp.py]
    Monitors[3 Surround monitors]
    DBox[D-BOX LiveMotion]
    Wheel --> Sender
    Cams --> Monitors
  end
  subgraph net [Tailscale]
    C5006[UDP 5006 control]
    V900x[UDP 9001-9003 JPEG]
    H5007[UDP 5007 IMU]
    S5008[UDP 5008 status]
  end
  subgraph vehicle [Jetson Orin]
    Ctrl[orin_vehicle_controller.py]
    CamSend[orin_camera_sender_udp.py]
    Imu[imu_bridge.py]
    Teensy[Teensy 4.1]
    Motors[Spark MAX motors]
    ZED[ZED X x3 ROS2]
    Xsens[Xsens IMU]
    Sender --> C5006 --> Ctrl --> Teensy --> Motors
    ZED --> CamSend --> V900x --> Cams
    Xsens --> Imu --> H5007 --> DBox
    Ctrl --> S5008
  end

# Port map

| Direction | Port | Protocol | Payload |
| --- | --- | --- | --- |
| Windows → Jetson | 5006 | UDP | steering,throttle,brake,mode |
| Jetson → Windows | 5007 | UDP | IMU binary @ ~50 Hz |
| Jetson → Windows | 5008 | UDP | Vehicle controller status |
| Jetson → Windows | 9001 / 9002 / 9003 | UDP | Chunked JPEG (primary) |
| Jetson → Windows | 9001 / 9002 / 9003 | TCP | JPEG (legacy) |

Primary video: chunked JPEG over UDP (~1280×720 @ 30 fps). TCP JPEG and H.264 RTP remain fallbacks.

# Control path

Thrustmaster (SDL2)
  → simchair_sender_dbox.exe
  → Tailscale UDP :5006
  → orin_vehicle_controller.py  (default --duty → UL/UR)
  → Teensy /dev/ttyACM0 @ 115200
  → REV Spark MAX → motors

Mode: 0=stop, 1=drive

Never send Teensy S; use UL0 UR0 / L0 R0

WebUI and vehicle controller are mutually exclusive on /dev/ttyACM0

# Video path (primary)

ZED X → ROS2 raw Image
  → orin_camera_sender_udp.py  (resize + JPEG encode + UDP chunks)
  → Tailscale UDP :9001-9003
  → windows_camera_receiver_udp.py
  → monitors at x=0, 1920, 3840

Target: 1280×720 @ 30 fps, JPEG q≈85. Chunks ≤1200 bytes (Tailscale MTU).

# Haptic path (ready for E2E verify)

Xsens IMU → ROS2 /imu/data
  → imu_bridge.py → UDP :5007 @ 50 Hz
  → Windows washout / D-BOX listener

# Key software

| Role | File |
| --- | --- |
| Wheel + D-BOX sender | windows/simchair_sender_dbox.exe |
| UDP JPEG display | windows/windows_camera_receiver_udp.py |
| One-shot Windows launch | windows/start_all.bat (VIDEO_MODE=udp) |
| Full Jetson stack | jetson/orin_launch_all.sh |
| UDP → Teensy | jetson/orin_vehicle_controller.py |
| UDP JPEG sender | jetson/orin_camera_sender_udp.py |
| IMU → UDP 5007 | jetson/imu_bridge.py |

# Must-know Jetson constraints

unset DISPLAY before ZED (empty DISPLAY= still breaks Argus)

ROS_DOMAIN_ID=42, CycloneDDS, source IGVC install

Use /usr/bin/python3

ZED: publish_tf:=true publish_urdf:=true; stagger ~25 s

Never modify /home/dinosaur/IGVC/

Tailscale MTU 1280: UDP JPEG chunks ≤1200 bytes

IPs: Windows 100.91.25.107 · Jetson 100.93.121.3

See also: SYSTEM_ARCHITECTURE.md · LAUNCH_PROCEDURE.md