# Challenges and Solutions: Marvin Sim Chair

Solved issues and architecture choices that keep the demo path reliable. Open problems are listed separately at the end.

# Solved: Challenge → Root cause → Solution

| Challenge | Root cause | Solution |
| --- | --- | --- |
| ZED crash / EGL BadParameter | DISPLAY set, or export DISPLAY= (empty string still breaks Argus) | Real unset DISPLAY on its own line before every ZED launch |
| ZED hangs: “Waiting for valid static transformations” | publish_tf:=false / publish_urdf:=false while tracking stays on in IGVC yaml | Launch with publish_tf:=true publish_urdf:=true (+ serial + yaml) |
| CAMERA STREAM FAILED TO START | IGVC ZED watchdogs and teleop ZEDs both owning cameras | sudo pkill -f zed_camera.launch.py, restart Argus; only one stack |
| Multi-camera Argus crash | Three ZEDs started at once | Stagger launches with 25 s sleep between cameras |
| Black / missing OpenCV windows on Windows | cv2 calls from background threads | Receive on threads; imshow only on main thread via queues |
| Wrong / no video from ROS | Subscribed to incorrect topic names | Use /zed_*/zed_node/rgb/color/rect/image (+ compressed variant for sender) |
| Throttle/brake and steering felt inverted | Sign error in mix | (throttle - brake) and negated steering scale |
| Port already in use on relaunch | Previous process still bound | sudo pkill -f <script> before restart |
| GStreamer RTSP unreliable / black frames | Thread-safety with GstRtspServer; env/topic mistakes | Prefer JPEG; TCP then UDP chunked; GStreamer/RTP secondary |
| Conda breaks ROS Python imports | python3 points into conda | Always /usr/bin/python3 on Jetson |
| Velocity cmds ack but no motion | Spark closed-loop / encoder path | Default vehicle controller to `--duty` |
| WebUI dead after simchair Ctrl+C | Restored actuator_node only | Restart full `avros-webui` service |
| Dual Teensy owners | WebUI + vehicle controller | Exclusive ownership; launch_all stops WebUI when driving |

# Architecture decisions (why we built it this way)

| Decision | Why |
| --- | --- |
| JPEG over UDP (chunked) as primary video | Raises FPS/quality vs TCP 15 fps path; no TCP HOL blocking; stays OpenCV-simple (no GStreamer required) |
| TCP JPEG kept as fallback | Proven 2026-08-04 path if UDP/firewall issues |
| Raw re-encode at 1280×720 q85 | Control resolution/quality independent of ROS compressed downscale |
| Tailscale for all links | Works without campus LAN; machines on independent internet |
| Hardcoded 25 s ZED stagger | Fixed sleep more reliable than topic-ready polling |
| Duty-mode drive default | Velocity setpoints left Sparks purple; UL/UR moves kart |
| UDP control CSV on :5006 | Low overhead; Jetson watchdog zeros on silence |

MTU note: Tailscale MTU is 1280: UDP JPEG chunks ≤1200 bytes.

# Still open (do not present as solved)

| Issue | Symptom | Suspected cause | Next step |
| --- | --- | --- | --- |
| Motor degradation | Fine at first launch; later hard to move / unresponsive | Teensy firmware limiter or Spark current limit | A/B with phone-joystick Teensy firmware |
| Kart tilts right | Drift/tilt right despite symmetric L/R commands | Hardware / mismatched Spark configs | Compare left vs right Spark parameters |
| IMU → D-BOX E2E | imu_bridge.py sends UDP 5007 | Washout/listener path not demo-verified | Live run: confirm chair motion from vehicle IMU |
| Tailscale DERP | Often relayed (lax), not direct | NAT/firewall | Aim for direct before latency claims |

See also: KNOWN_ISSUES_AND_HISTORY.md · TARGET_ARCHITECTURE.md