# Results and Metrics: Marvin Sim Chair

Baseline recorded 2026-07-27 (Ryan Simpson / AVL). TCP JPEG re-verified 2026-08-04. UDP JPEG primary path introduced 2026-08-07. Numbers below are lab measurements: do not invent values for TBD rows.

Link: Windows 100.91.25.107 ↔ Jetson 100.93.121.3 (Tailscale)

# Verified capabilities

| Feature | Result | Notes |
| --- | --- | --- |
| Sim-chair wheel/pedal input | Working | SDL2 → simchair_sender_dbox.exe |
| UDP control to Jetson | Working | Port 5006 over Tailscale |
| Vehicle controller → Teensy | Working | Serial 115200; default --duty (UL/UR) |
| Three-camera JPEG streaming | Working | UDP 9001-9003 chunked JPEG (TCP legacy) |
| D-BOX SDK on Windows | Working | LiveMotion app key integrated |
| ZED multi-camera launch | Working | 25 s stagger; TF/URDF flags required |
| Vehicle IMU → D-BOX chair | Not yet verified E2E | Bridge script ready (imu_bridge.py) |

# Network (baseline)

| Metric | Value | Notes |
| --- | --- | --- |
| Tailscale path | DERP relay (lax) | Target: direct (relay adds ~50-100 ms) |
| Tailscale ping RTT | 88-112 ms (10 samples) | From sim-chair, 2026-07-27 |
| Direct peer connection | No (at baseline) | Fix NAT/firewall before latency demo claims |

# Video

| Metric | TCP baseline (2026-08-04) | UDP primary (2026-08-07) | Next target |
| --- | --- | --- | --- |
| Resolution | ~960×600 display | 1280×720 encode target | 1920×1200 / H.265 |
| Frame rate | 15 fps cap | 30 fps target | 30+ sustained |
| Codec / transport | JPEG / TCP | JPEG / UDP (chunked) | H.265 / RTP-UDP |
| Glass-to-glass latency | TBD | TBD | <150 ms median |
| Total bandwidth (3 cams) | TBD | TBD | <25 Mbps |

# Control (UDP :5006)

| Metric | Baseline | Target |
| --- | --- | --- |
| Packet format | steering,throttle,brake,mode | Add seq, timestamp_ms |
| Send rate | ~20 Hz (SDL loop) | 50 Hz fixed |
| Jetson watchdog | 400 ms | 300 ms + latched STOPPED |
| Teensy watchdog | None | 250 ms silence → zero motors |
| Failsafe (cable pull on stands) | TBD | Stop <300 ms; re-arm required |

# Haptics

| Metric | Baseline | Target |
| --- | --- | --- |
| IMU topic | /imu/data (Xsens MTi-680G) | Same |
| Bridge rate | Script @ 50 Hz design | Confirmed 50 Hz on wire |
| IMU → chair path | Not E2E verified | UDP 5007 → washout → D-BOX |
| Packet-loss chair behavior | N/A | Ramp to neutral ≥1 s |

# How we measure (for filling TBDs)

Glass-to-glass: phone stopwatch in front of a ZED lens; photograph phone + operator monitor together; 10 samples → report median.

Tailscale path:

tailscale ping 100.93.121.3
tailscale status   # must say "direct", not relay

IMU rate (Jetson): ros2 topic hz /imu/data with domain 42 + CycloneDDS env loaded.

See also: MEASUREMENTS.md · KNOWN_ISSUES_AND_HISTORY.md