# Future Work: Marvin Sim Chair

Priority-ordered next steps and the upgrade phases that move the stack from “demo-capable” to industry-style teleoperation.

# Priority order (do these first)

Motors: re-verify spin / Spark / measured RPM (WebUI + sim-chair); chase degradation and right-tilt

IMU → D-BOX end-to-end: imu_bridge.py exists; confirm chair motion from vehicle IMU on a live run

Video quality: UDP JPEG at 720p30 is in; next H.265 NVENC / ZED stream for bandwidth

Closed-loop velocity: restore reliable --velocity (encoders / Spark PID) so duty is not permanent

Latency: measure glass-to-glass on direct Tailscale; target <150 ms median

Keep mirrors in sync: edit in cursor-test\jetson\, push to ~/Documents/SimChair_Teleop/

# Upgrade phases (current → target)

| Subsystem | Current | Target | Direction |
| --- | --- | --- | --- |
| Video | Chunked JPEG/UDP, 1280×720 @ 30 | H.265 NVENC, RTP/UDP, ≥1080p | ZED stream / GStreamer RTP |
| Control | UDP CSV ~20 Hz; duty default | Seq + timestamps; reliable velocity PID | Failsafe + encoder bring-up |
| Haptics | Bridge script; E2E not verified | Classical washout; ramp to neutral on loss | UDP 5007 → washout → D-BOX |
| Motors | Duty works; velocity/purple issues; tilt | Sustained smooth closed-loop | Teensy/Spark A/B debug |
| Ops | Manual tmux; iptables lost on reboot | systemd + iptables-persistent | Demo reliability |

Biggest single win for video next: keep UDP JPEG as fallback; adopt hardware H.265 so bandwidth drops at higher resolution.

# Industry reference (what “done” looks like)

Commercial remote-driving stacks converge on:

Hardware-encoded video over UDP (lost packet = one frame, not a stall)

Fixed-rate control with sequence numbers and multi-layer watchdogs (stop in ~250-500 ms of silence)

Motion platforms with classical washout filtering

Supervised services that auto-restart and fail safe

Campus Tailscale is fine for demos; bonded cellular is the commercial WAN pattern.

# Integration milestones

| Milestone | Pass criteria |
| --- | --- |
| Haptics stands test | Brake pitch / cornering roll / bump heave felt in chair |
| Control failsafe | Cable pull → motors stop <300 ms; re-arm required |
| Video Phase 1 | ≥720p, ≥30 fps, glass-to-glass <150 ms, bandwidth <25 Mbps (3 cams) |
| Soak | ≥30 min simultaneous cameras + control + haptics without motor collapse |
| Ops | Cold boot → operational with zero manual tmux |

# Constraints that stay

Never modify /home/dinosaur/IGVC/

Teleop scripts: Jetson ~/Documents/SimChair_Teleop/ · Windows cursor-test\windows\

Prefer lab-accurate claims over symposium language until metrics are filled in MEASUREMENTS.md

See also: TARGET_ARCHITECTURE.md · KNOWN_ISSUES_AND_HISTORY.md · MEASUREMENTS.md