# Codebase & Handoff Documentation: Marvin Sim Chair

AVL Summer Cohort 2026 final deliverable

Updated: 2026-08-07

Audience: next AVL operators, mentors, and AI sessions continuing this work

This package covers repository structure, setup instructions, known issues, and future recommendations. Authoritative ops detail remains in the linked docs under docs/.

# 1. Repository structure

C:\Users\VRX 226\cursor-test\
├── LAUNCH.txt                 Short cheat sheet
├── windows\                   Windows runtime: LAUNCH FROM HERE
│   ├── simchair_sender_dbox.exe
│   ├── dbxLive64.dll / SDL2.dll
│   ├── windows_camera_receiver_udp.py  ← PRIMARY video (UDP JPEG)
│   ├── windows_camera_receiver.py      ← legacy TCP JPEG
│   ├── windows_zed_stream_receiver.py  ← experimental
│   ├── windows_gstreamer_rtp_receiver.py
│   ├── windows_control_status.py
│   ├── windows_imu_washout_test.py
│   ├── start_all.bat                   ← VIDEO_MODE=udp default
│   └── requirements.txt
├── jetson\                    Mirror of Jetson teleop scripts
│   ├── orin_launch_all.sh              ← full stack (UDP cams + --duty)
│   ├── orin_camera_sender_udp.py       ← PRIMARY video
│   ├── orin_camera_sender.py           ← legacy TCP
│   ├── orin_vehicle_controller.py      ← default --duty
│   ├── imu_bridge.py
│   └── simchair_drive.sh               ← restores full avros-webui
├── docs\                      Full documentation
│   ├── deliverables\          ← THIS cohort submission pack
│   ├── deck\                  Section briefs
│   ├── LAUNCH_PROCEDURE.md
│   ├── SYSTEM_ARCHITECTURE.md
│   ├── KNOWN_ISSUES_AND_HISTORY.md
│   ├── MEASUREMENTS.md
│   ├── TARGET_ARCHITECTURE.md
│   └── ...
├── CARS_Poster\               Symposium poster / Canva pack
└── _archive\                  Old mirrors / experiments: not day-to-day

Live Jetson path: ~/Documents/SimChair_Teleop/ (keep in sync with cursor-test\jetson\)

IGVC (read-only): /home/dinosaur/IGVC/: never modify

C++ / D-BOX sources (external): C:\Users\VRX 226\Test Workspace\

## Network identity

| Machine | Tailscale IP |
| --- | --- |
| Windows sim-chair | 100.91.25.107 |
| Jetson dinosaur | 100.93.121.3 |

## Ports

| Port | Proto | Role |
| --- | --- | --- |
| 5006 | UDP | Control Windows → Jetson |
| 5007 | UDP | IMU Jetson → Windows |
| 5008 | UDP | Status Jetson → Windows |
| 9001-9003 | UDP | Chunked JPEG video (primary) |
| 9001-9003 | TCP | JPEG video (legacy fallback) |

# 2. Setup instructions

## 2.1 One-time / environment

Windows

Install Python 3 + pip install -r windows\requirements.txt (opencv-python, numpy)

ZED SDK / GStreamer only if using experimental video modes

Confirm Tailscale connected; three monitors in Surround (offsets 0 / 1920 / 3840)

Thrustmaster drivers + D-BOX LiveMotion with APP_KEY CalPolyPomonaDevSim

DLLs (dbxLive64.dll, SDL2.dll) must sit next to simchair_sender_dbox.exe

Jetson

ROS 2 Humble + IGVC install already present under /home/dinosaur/IGVC/

Teleop scripts in ~/Documents/SimChair_Teleop/

Tailscale connected

Use /usr/bin/python3 (conda hijacks python3)

## 2.2 Every session: current demo path

Jetson

bash ~/Documents/SimChair_Teleop/orin_launch_all.sh

Preflight clears ZED contention + restarts Argus. Starts tmux teleop: firewall (TCP+UDP 9001-9003), sensors, ZEDs, UDP camera sender, vehicle `--duty`, IMU bridge. Stops WebUI while driving.

Stop: tmux kill-session -t teleop then sudo systemctl start avros-webui

Windows

cd "C:\Users\VRX 226\cursor-test\windows"
.\start_all.bat
# default VIDEO_MODE=udp → windows_camera_receiver_udp.py
# TCP fallback:  $env:VIDEO_MODE = "jpeg"; .\start_all.bat

## 2.3 ROS 2 env (any Jetson terminal touching ROS)

unset DISPLAY
source /home/dinosaur/IGVC/install/setup.bash
export RMW_IMPLEMENTATION=rmw_cyclonedds_cpp
export CYCLONEDDS_URI=file:///home/dinosaur/IGVC/install/avros_bringup/share/avros_bringup/config/cyclonedds.xml
export ROS_DOMAIN_ID=42

ZED launch must use publish_tf:=true publish_urdf:=true and real unset DISPLAY (never export DISPLAY=).

## 2.4 Sync scripts after edits

Edit in cursor-test\jetson\, then copy to ~/Documents/SimChair_Teleop/ on Marvin (SCP or Tailscale HTTP file server if keys missing).

Full procedure: ../LAUNCH_PROCEDURE.md · root ../../LAUNCH.txt

# 3. Known issues

## Working

Sim-chair wheel/pedal → UDP → Teensy (--duty)

Three-camera UDP JPEG streaming (2026-08-07); TCP path kept as fallback

D-BOX SDK integration on Windows

ZED multi-cam launch with stagger + TF flags

simchair_drive.sh restores full avros-webui

## Open

| Issue | Notes |
| --- | --- |
| Velocity / closed-loop drive | Often Sparks purple; use --duty |
| Motor degradation after sustained use | Suspect Teensy firmware / Spark limits |
| Kart tilts/drifts right | Suspect hardware / Spark config mismatch |
| IMU → D-BOX end-to-end | imu_bridge.py ready; chair cueing not demo-verified |
| Tailscale often DERP not direct | Hurts latency budget |
| Glass-to-glass / bandwidth | Still TBD in measurements |
| iptables not persistent | Re-apply each boot (or use launch_all) |

## Do not regress (lessons learned)

Real unset DISPLAY before ZED

TF/URDF true or no image frames

One ZED stack only; 25 s stagger

OpenCV main thread on Windows

/usr/bin/python3 on Jetson

Never send Teensy S stop: use UL0 UR0 / L0 R0

Only one Teensy owner; restore full WebUI service after sim chair

Never write inside /home/dinosaur/IGVC/

Full history: ../KNOWN_ISSUES_AND_HISTORY.md

# 4. Future recommendations

Priority order:

Motors: A/B Teensy firmware; verify meas RPM; fix right-tilt Spark configs

Haptics E2E: live IMU → washout → D-BOX; ramp to neutral on packet loss

Video: H.265 NVENC / RTP next; keep UDP JPEG as fallback

Latency: Tailscale direct; glass-to-glass <150 ms median

Control hardening: seq + timestamps, 50 Hz, 300 ms latched STOPPED, Teensy silence watchdog

Ops: systemd units, iptables-persistent, cold-boot without manual tmux

Soak: ≥30 min simultaneous video + control + haptics

Roadmap detail: ../TARGET_ARCHITECTURE.md · metrics log: ../MEASUREMENTS.md

# 5. File roles (quick)

| File | Purpose |
| --- | --- |
| simchair_sender_dbox.exe | Wheel/pedals + D-BOX sender |
| windows_camera_receiver_udp.py | Primary UDP JPEG display |
| windows_camera_receiver.py | Legacy TCP JPEG display |
| orin_launch_all.sh | Full Jetson teleop stack |
| orin_vehicle_controller.py | UDP → Teensy (default --duty) |
| orin_camera_sender_udp.py | Primary UDP JPEG sender |
| orin_camera_sender.py | Legacy TCP JPEG sender |
| imu_bridge.py | IMU → UDP 5007 @ 50 Hz |
| simchair_drive.sh | Borrow Teensy; restart avros-webui |

Inventory: ../FILES_REFERENCE.md

# 6. Safety & constraints for next operators

Spotter at the kart for any powered demo

Start in stop mode; enable drive only when clear

Prefer stands for early motor tests

Sim-chair and Chrome WebUI must not both own /dev/ttyACM0

Reliability beats new features for demos

AI/operator mission context: ../HANDOFF_GUIDE.md

# 7. Cohort deliverables cross-links

| Deliverable | Path |
| --- | --- |
| Technical report | TECHNICAL_REPORT.md |
| Slide deck (Marp) | SLIDE_DECK.md |
| Slide deck (HTML) | slide_deck.html |
| Demo video script | DEMO_VIDEO_SCRIPT.md |
| Demo video file | Record → DEMO_VIDEO.mp4 (not in repo until filmed) |
| This handoff | HANDOFF_PACKAGE.md |